--------------------------- SPECIFICATION ---------------------------

1. TITLE You can form the title from the keywords below. Try not to go beyond two  sentences.

The Impact of Flicker Frequency and Modulation Depth on Drivers’ Lane-Change  Decision-Making: An Exploratory Study of Camera Monitor System Displays

2. Keywords: choose, in order of importance, four keywords that describe your  project.

Camera Monitor System; Flicker Frequency; Modulation Depth; Lane-Change  Decision-Making; Last Safe Gap; Visual Search Behavior; Cognitive Load;  Driving Safety; Automotive Design

3. Your Research Question: Main RQ: How do CMS display flicker frequency and modulation depth affect drivers’ lane-change decision-making? SRQ 1: How do different flicker frequencies, compared to a stable display condition, influence drivers’ lane‑change decision‑making? SRQ 2: How do flicker frequency and modulation depth affect drivers’ visual search behavior and subjective cognitive load during lane-change decision-making? SRQ 3: How do flicker frequency and modulation depth interact to influence drivers' lane-change decision-making?

------------------------------------------------------------------------------------------------------------------------------------------------------------ Project Specification Template v1.3 C. Peters

4. Your Hypothesis (claim) and reasoning/evidence: Hypothesis 1: ● Flicker frequency will significantly affect drivers’ lane-change decision-making performance. Compared to the stable display (0 Hz), flicker conditions, especially mid-range frequencies (12.5 Hz), are expected to produce greater deviations in lane-change decisions. ● Reasoning ：Frequencies around 10–20 Hz are closer to the range where flicker is perceptually salient, potentially increasing perceptual instability. Flicker interferes with temporal visual processing and motion perception. Hypothesis 2: ● Higher flicker frequency and greater modulation depth will increase visual search effort and subjective cognitive load compared to the stable display condition. ● Reasoning: Greater modulation depth increases luminance fluctuation, reducing display stability. This perceptual instability requires additional attentional resources to extract motion cues, thereby increasing visual search demand and cognitive workload. Hypothesis 3: ● There will be a significant interaction effect between flicker frequency and modulation depth on drivers’ lane-change decision-making, with higher modulation depth amplifying the influence of flicker frequency on decision-making. ● Reasoning: The perceptual impact of flicker depends jointly on temporal frequency and luminance modulation amplitude. When both parameters reach perceptually disruptive levels, their combined effect may significantly impair motion estimation and decision accuracy.

* Remember: a thesis is a scientific (evidence-based) argument. It’s not (just) a factual description of work  done, but nor is it perfection. It often raises more questions than it answers.

------------------------------------------------------------------------------------------------------------------------------------------------------------ Project Specification Template v1.3 C. Peters

5. Implementation/stimuli

What technical work will you do in order to create the stimuli and show them to participants?

Table includes alternatives. Use strikethrough when you have decided against an alternative or  bold font to indicate you have made a choice. Normal text indicates possibilities in your mind.

| Interaction technologies | Display technologies | Graphics and audio | Scenario and assets |
| --- | --- | --- | --- |
| Video-based experimental task | 65-inch LG OLED 4K TV (60 fps) 4K resolution (3840×2160) | Pre-recorded real traffic video clips | Real-world highway environment (E4 highway, Sweden) |
| Steering wheel (Thrustmaster T248) for LSG response input | Controlled laboratory lighting (≈450 lux ) | Audio cue embedded in video (attention signal) | High camera height (2.3 m) only & Fixed FOV: 112° |
| Mouse and keyboard for questionnaire responses | Fixed viewing distance (1.5 × screen height) |  | 30 videos 1 stable condition with 0 Hz 3 flicker frequencies × 3 modulation depths = 9 flicker conditions 10 conditions, and with 3 scenes, this results in 30 videos. |

● Note: You should change these headings to relate to your specific project type. Only  include important components of your project. Examples of other important headings that  could apply to your project include "algorithms","datasets", etc.

------------------------------------------------------------------------------------------------------------------------------------------------------------ Project Specification Template v1.3 C. Peters

6. Minimal/first working prototype

Define, as specifically as possible, what your minimal working prototype should do and what  features it needs to have to do it. You can also include stretch features i.e. those you may not get  to implementing. It should be as simple as possible while doing something useful.

What it will do: The system will present 30 pre-recorded driving videos stimuli consisting of a forward driving view and a CMS rear-view display. The video stimuli were programmatically manipulated across 3 road scenes to simulate 10 conditions: a stable baseline (0 Hz) and nine flicker configurations involving three frequencies (8.33, 12.5, and 25 Hz) crossed with three modulation depths (40%, 60%, and 80%). Lane-change decision-making is operationalized using the Last Safe Gap (LSG) method. During each trial, participants are required to indicate the 'Last Safe Gap' by pressing the left paddle behind the steering wheel at the final moment they deem a lane-change maneuver safe. The system records trial conditions, participant response inputs, and subjective ratings. These data points enable a quantitative analysis of how display flicker parameters, like frequency and modulation depth, influence drivers' lane-change judgment margins and overall decision-making stability. The prototype focuses on stimulus control, randomized presentation, and response logging, while relying on pre-generated video stimuli rather than a full driving simulator.

------------------------------------------------------------------------------------------------------------------------------------------------------------ Project Specification Template v1.3 C. Peters

7. Evaluation

Write evaluation possibilities. These will usually be either comparisons with other  algorithms/implementations, or parameter variations within a single algorithm. Be specific  and link each evaluation to your research questions above.

| Evaluation Label | Measures / comparisons | Priority | Research Question Links |
| --- | --- | --- | --- |
| LSG decision performance | Compare LSG decision timing across flicker frequencies (0, 8.33, 12.5, 25 Hz) and modulation depths (40%,60%,80%). Analyze how these parameters affect the moment participants choose to change lanes. | High | RQ1, RQ3 |
| Visual attention behaviour (Eye-tracking) | Eye-tracking metrics including fixation duration, fixation count, and gaze transitions between the forward road view and CMS display under different flicker conditions. | High | RQ2 |
| Subjective workload & perception | After each stimulus, we use 7-point Likert scales to assess Visual Comfort, Decision Certainty. Two core dimensions of the NASA TLX, Mental Demand and Effort, are used because they most directly reflect the cognitive workload associated with visual processing and decision-making tasks in this experiment. (*Physical demand and temporal demand are minimal in the video based task, and therefore were not included.) | High | RQ2 |

Section 8 is only for user studies

------------------------------------------------------------------------------------------------------------------------------------------------------------ Project Specification Template v1.3 C. Peters

8. Experiment Design, Measures and Analysis

| Stimuli ordering and order effects | Conditions / variables | Participant task | Scenario | Total time of experiment | Experiment location |
| --- | --- | --- | --- | --- | --- |
| A within subject design is conducted. Each participant completes 30 trials. The order of video presentation is randomized to reduce order effects and learning effects. Practice trials are provided before the formal experiment. A short break is inserted after 15 trials to reduce visual fatigue and discomfort caused by flicker. | 1 stable condition with 0 Hz 3 flicker frequencies × 3 modulation depths = 9 flicker conditions 10 conditions, and with 3 scenes, this results in 30 videos. | Participants watch each driving video and indicate the last safe opportunity to change lanes (Last Safe Gap, LSG) by pressing the left paddle behind the steering wheel. After each trial, participants complete a short rating questionnaire. Eye-tracking data are recorded throughout the experiment. | Pre-recorded highway driving videos are used, showing both the forward driving view and a CMS rear-view display. Flicker conditions are simulated in the CMS display. The camera height and field of view are fixed (High position, 112° FOV). | Each trial lasts about 30 seconds (including video playback and rating time). 30 trials take approximately 20 minutes. Including instructions, eye-tracking calibration, and interviews, the total experiment duration is approximately 1h. | The experiment is conducted in a controlled laboratory environment with stable ambient lighting (approximately 450 lux). Videos are displayed on a 65-inch 4K OLED screen, and an eye tracker is used to record participants’ visual behavior. |

4 Question after each video trial:(scale :1-7)

1. How visually comfortable was the display?

2. How mentally demanding was this task?

3. How hard did you have to work to maintain your performance?

4. How certain are you in your decision?

------------------------------------------------------------------------------------------------------------------------------------------------------------ Project Specification Template v1.3 C. Peters

For ‘measures’, it is useful to add a link to any custom questionnaires (as Google Docs, forms,  etc) you are working on.

| Standard questionnaires | Custom questions | Performance measures |
| --- | --- | --- |
| Background questionnaire Post-experiment questionnaire | Trial-level ratings after each video assessing decision confidence. Post-experiment interview questions regarding participants’ strategies and experiences. | LSG decision timing, converted to Time-to-Contact (TTC). Eye-tracking metrics, including fixation duration, fixation count, and gaze distribution between the forward road view and the CMS display. |

Analysis

What statistical techniques will be used to analyse your results?

| Graphs | Standard deviation | Significance (p/t values) | Other | General text |
| --- | --- | --- | --- | --- |
| Bar charts and box plots will be used to visualize TTC, eye tracking metrics, and subjective ratings (confidence and visual comfort) under different flicker frequency and modulation depth conditions. Eye tracking data will be presented using gaze distribution plots. | Means and standard deviations will be calculated for TTC, eye tracking metrics, and subjective ratings across experimental conditions. | Two-way repeated measures ANOVA will be used to analyze the effects of flicker frequency and modulation depth. Greenhouse-Geisser corrections will be applied to adjust for violations of the sphericity assumption. | Interaction effects between flicker frequency and modulation depth will be analyzed. In the case of significant main effects or interactions, post-hoc pairwise comparisons will be performed using Holm-Bonferroni corrections (Holm, 1979) to control for multiple comparisons. | The statistical analysis aims to evaluate how different CMS flicker conditions influence drivers’ LSG decision making, visual attention allocation, and subjective perception. |

------------------------------------------------------------------------------------------------------------------------------------------------------------ Project Specification Template v1.3 C. Peters

9. Project/scenario Delimitations

Sometimes you can better understand what something is by knowing what it is not. List any  constraints or delimitations on the domain of your work.

This study focuses on the effects of flicker characteristics in Camera Monitor System (CMS) displays on drivers’ lane‑change decision‑making. The experiment is conducted in a controlled laboratory environment using pre-recorded video stimuli, rather than in real driving conditions or a full driving simulator. The study only examines two intrinsic flicker parameters: flicker frequency and modulation depth. Other display characteristics, such as brightness, contrast, and resolution, are kept constant and are not included as experimental variables. In addition, the experimental task is limited to LSG decision-making scenarios and does not include other driving behaviors or tasks, such as distance estimation, full driving performance, or vehicle control. The study involves approximately 25 participants; therefore, the results should be interpreted as exploratory findings and may not be fully generalizable to all driving situations.

10. Core references

No more than three key references that your implementation, methods and/or evaluation are  heavily based upon.

| Author | references |
| --- | --- |
| Thulinsson & Söderlund (2024) | Mirror, Mirror on the Car: An Explorative Study on how CMS Perspective Impact Driver Depth Perception and Decision Making |
| Pifferi (2025). | The impact of age on drivers' perception and decision making: An exploratory study of Camera Monitoring Systems |
| Bullough et al. (2013) | Visual Task Performance and Perceptions of Lighting Quality Under Flickering Illumination |

------------------------------------------------------------------------------------------------------------------------------------------------------------ Project Specification Template v1.3 C. Peters

11. Time plan

Make sure to include your intended thesis presentation date. From there work backwards. If you  are working on a thesis, then a draft should be submitted to the examiner at least one week  before the presentation date. There will therefore need to be sent to the supervisor a few weeks  before that. These are all dates that can be estimated and added in.

| Date | Milestone |
| --- | --- |
| Jan 2026 | Literature Study |
| 27 February 2026 | Living Specification &Individual Plan iteration |
| 14 April 2026 | Experimental setup fully implemented |
| 17 April 2026 | Pilot testing completed and adjustments finalized |
| 8 May 2026 | Conduct experiment and collect data with 25 participants |
| 15 May 2026 | Statistical analysis finalized |
| 30 May 2026 | Full thesis draft submitted to supervisor |
| 3 June 2026 | Thesis submission to examiner one week before intended presentation date |
| 10 June 2026 | Oral thesis presentation |

------------------------------------------------------------------------------------------------------------------------------------------------------------ Project Specification Template v1.3 C. Peters