Eye-Tracking Data Export

Parameter Definitions and Computation Logic

CMS Flicker Study — Dual-Machine Setup (A: Tobii Pro Lab / B: Video Playback)

# 1  Overview

This document describes every column in the exported eye-tracking analysis spreadsheet. The data is generated by a Python script that (1) parses the B-machine debug log to extract per-video playback timestamps, (2) reads the Tobii Pro Lab gaze-data export (.tsv), (3) automatically corrects any system-clock offset between the two machines, and (4) segments the gaze stream by video clip to compute the metrics below.

All time-based metrics use milliseconds unless otherwise noted. Pupil diameters are in millimetres. Gaze coordinates follow Tobii's screen-based coordinate system (pixels, origin at top-left).

# 2  Experimental Setup

Machine A runs Tobii Pro Lab in Screen Recording mode, recording gaze data from a Tobii Pro eye tracker. Machine B plays 30 video clips (4K, 50 fps, ~21 s each) through a custom DirectX-based renderer and logs precise playback timestamps. Both machines output to the same physical display; the video signal source is switched between them. The eye tracker is calibrated while Machine A drives the display.

Because Machine A records a screen capture of its own desktop (which shows nothing during B-machine playback), the recorded video is blank. However, gaze coordinates remain valid because the eye tracker tracks physical eye position relative to the monitor, independent of display content.

# 3  AOI Definition

A single rectangular Area of Interest (AOI) is defined in Tobii Pro Lab, covering the Camera Monitor System (CMS) rear-view display in the lower-left corner of the driving scene. The AOI is drawn on a reference frame within Pro Lab and exported as the "AOI hit" column (1 = gaze inside AOI, 0 = gaze outside AOI, −1 = AOI not active at this timestamp).

For timestamps where the AOI is not active (hit = −1), a coordinate-based fallback is applied: the CMS bounding rectangle is estimated from the 0.5th–99.5th percentile of gaze coordinates of confirmed AOI-hit samples, with a 10 % margin.

# 4  Clock Synchronisation

The two machines may have different system clocks. The script determines the offset automatically by testing candidate offsets from −600 s to +600 s. For each candidate, it counts how many AOI-hit gaze samples land within the 30 video-playback windows. The offset producing the maximum count is selected (coarse pass at 5 s, fine pass at 0.1 s). Once corrected, B-machine timestamps are mapped into Tobii's recording-relative time axis.

# 5  Parameter Reference

## 5.1  Metadata & Session Information

| Parameter | Description | Computation Logic |
| --- | --- | --- |
| participant | Participant identifier | Extracted from the Tobii Pro Lab export file (Participant name field). |
| video_index | Playback order index (0–29) | Sequential index from B-machine debug log, representing the order in which videos were presented to this participant. |
| video_name | Video clip filename | Parsed from debug log entry "Starting to play video #N: <path>". The directory path and .bgr extension are stripped, leaving only the clip name which encodes the experimental condition (scene variant, flicker frequency, modulation depth). |
| playback_start | Video playback start time (HH:MM:SS.mmm) | Extracted from B-machine log line "[timestamp] Video #N starting playback". This is the B-machine system clock time. |
| playback_end | Video playback end time (HH:MM:SS.mmm) | Extracted from B-machine log line "[timestamp] Video #N playback completed normally". |
| clock_offset_s | Clock offset between A-machine and B-machine (seconds) | Automatically determined by brute-force search over the range ±600 s in two passes: (1) coarse search at 5 s resolution, (2) fine search at 0.1 s resolution. The offset that maximises the number of AOI-hit samples falling within video playback windows is selected. Positive value means B-machine clock is behind A-machine. |

## 5.2  Data Quality

| Parameter | Description | Computation Logic |
| --- | --- | --- |
| total_duration_ms | Segment duration (ms) | Difference between the last and first Recording timestamp values within the segment: (ts_last − ts_first) / 1000. |
| valid_samples | Number of valid gaze samples | Count of Eye Tracker samples where both Gaze point X and Gaze point Y are non-null. |
| valid_ratio | Proportion of valid gaze samples (%) | valid_samples / total_samples × 100. |
| eyes_not_found_ratio | Proportion of EyesNotFound samples (%) | Count of samples with Eye movement type = "EyesNotFound" / total_samples × 100. High values indicate tracking loss (e.g., blinks, head movement). |

## 5.3  CMS Dwell Time

| Parameter | Description | Computation Logic |
| --- | --- | --- |
| dwell_time_cms_ms | Total gaze dwell time on CMS area (ms) | For each valid gaze sample classified as inside the CMS AOI, the inter-sample interval (timestamp difference to preceding sample) is accumulated. AOI membership is determined by the Tobii Pro Lab AOI hit column (value = 1). For samples where the AOI is inactive (value = −1), a coordinate-based fallback is used: the CMS bounding box is inferred from the 0.5th–99.5th percentile of Gaze point X/Y of confirmed AOI-hit samples, expanded by 10 %. |
| dwell_ratio_cms | CMS dwell time as percentage of valid viewing time (%) | dwell_time_cms_ms / total_valid_time × 100. |
| dwell_time_road_ms | Total gaze dwell time on road area (ms) | Complement of CMS dwell: total valid inter-sample time minus CMS dwell time. |
| dwell_ratio_road | Road dwell time as percentage (%) | 100 − dwell_ratio_cms. |

## 5.4  Pupil Diameter

| Parameter | Description | Computation Logic |
| --- | --- | --- |
| pupil_diameter_mean | Mean filtered pupil diameter (mm) | Arithmetic mean of the Tobii "Pupil diameter filtered" column across all valid samples in the segment. The filtered value is Tobii's built-in noise-reduced estimate combining left and right eyes. |
| pupil_diameter_std | Standard deviation of filtered pupil diameter (mm) | Standard deviation of Pupil diameter filtered across all valid samples. |
| pupil_diameter_cms | Mean pupil diameter while fixating on CMS (mm) | Mean of Pupil diameter filtered for samples classified as inside the CMS AOI. |
| pupil_diameter_road | Mean pupil diameter while fixating on road (mm) | Mean of Pupil diameter filtered for samples classified as outside the CMS AOI. |

## 5.5  Fixation Metrics

| Parameter | Description | Computation Logic |
| --- | --- | --- |
| fixation_count_total | Total number of fixations | Fixations are identified by Tobii's I-VT (velocity-threshold) filter. Each unique Eye movement type index with Eye movement type = "Fixation" is counted once. |
| fixation_count_cms | Number of fixations on CMS | A fixation is classified as CMS if >50 % of its constituent samples fall inside the CMS AOI (majority-vote rule). This avoids spurious classification from gaze noise at AOI boundaries. |
| fixation_count_road | Number of fixations on road | fixation_count_total − fixation_count_cms. |
| fixation_duration_mean_ms | Mean fixation duration across all fixations (ms) | Mean of Eye movement event duration for all unique fixations in the segment. |
| fixation_duration_cms_mean_ms | Mean fixation duration on CMS (ms) | Mean duration of fixations classified as CMS. |
| fixation_duration_road_mean_ms | Mean fixation duration on road (ms) | Mean duration of fixations classified as road. |

## 5.6  Gaze Transitions

| Parameter | Description | Computation Logic |
| --- | --- | --- |
| transition_count | Number of gaze transitions between CMS and road | Computed at the fixation level: fixations are sorted chronologically, each classified as CMS or road (majority vote). A transition is counted each time consecutive fixations change category (CMS→road or road→CMS). This fixation-level approach filters out noise-induced boundary crossings that inflate sample-level counts. |

## 5.7  First Fixation on CMS

| Parameter | Description | Computation Logic |
| --- | --- | --- |
| first_fixation_cms_time_ms | Time to first fixation on CMS (ms) | Recording timestamp of the earliest CMS-classified fixation sample minus the segment start timestamp, converted to milliseconds. Value of −1 indicates no CMS fixation occurred during the entire video clip. |

## 5.8  Saccade Metrics

| Parameter | Description | Computation Logic |
| --- | --- | --- |
| saccade_count | Total number of saccades | Count of unique Eye movement type index values with Eye movement type = "Saccade". This includes all saccades (within-road, within-CMS, and cross-region). |
| saccade_duration_mean_ms | Mean saccade duration (ms) | Mean of Eye movement event duration for all unique saccades. |

# 6  Notes

Fixation and saccade events are identified by Tobii Pro Lab's built-in I-VT (Fixation) filter applied during data export. The velocity threshold and other filter parameters are set within Pro Lab before export.

Transition count is computed at the fixation level rather than the raw-sample level. This is critical because gaze coordinates near AOI boundaries exhibit noise that produces spurious in/out crossings at the sample level (~120 Hz). The fixation-level approach groups samples into stable gaze events first, classifies each event by majority vote, and then counts category changes between consecutive events.

Pupil diameter values use Tobii's filtered estimate, which combines both eyes and applies noise reduction. Pupil diameter is a commonly used proxy for cognitive load, but is also affected by luminance. Since CMS and road regions may differ in brightness, between-region pupil comparisons should be interpreted with caution.

The clock_offset_s value is recorded per participant for transparency and reproducibility. A value near zero confirms that both machines were time-synchronised (e.g., via NTP). Large offsets indicate clock drift and are automatically compensated.